OTDR (Optical Time-Domain Reflectometer)

- Dynamic Range: Minimum 40dB (to ensure signal-to-noise ratio sufficiency over the 45 km span)

- Wavelength: Include 1550 nm

- Event Dead Zone: smaller then 1meter

- Attenuation Dead Zone: smaller than 5 meters

- Connectivity: Ethernet (RJ45) or USB interface for remote control.Software/API: Must support remote command-line control (SCPI commands) or provide a documented API for integration with Linux-based systems (Raspberry Pi)

- Pulse Width: The OTDR must support selectable pulse widths ranging from at least 10ns up to 5µs, with the capability to perform automated measurements at 1µs or higher to ensure signal propagation over the 45 km link

- File Format: Output must support the universal .sor (Telcordia/Bellcore) format

Launch Cable (Pulse Suppressor)

- Length: 1 km (to effectively bypass the OTDR's dead zone)

- Connectors: SC/APC (Angled Physical Contact) connectors on both ends

- Housing: Ruggedized spool or box enclosure for field protection

Raspberry Pi

- Unit: Raspberry Pi 4 Model B or Raspberry Pi 5 (for enhanced processing power)

- Network Interface:

  - Integrated Ethernet Port: Reserved for the direct connection to the OTDR (Static IP)

  - Secondary Interface: USB 3.0 to Gigabit Ethernet Adapter (must be Linux/Raspbian compatible) for connection to the local router/Internet

- Storage: Industrial-grade MicroSD card (High Endurance, e.g., 32GB/64GB) to ensure data reliability under continuous operation

- Enclosure: IP65-rated outdoor enclosure with adequate heat dissipation or active ventilation

- Power Management: Stable power supply unit (PSU) with surge protection suitable for outdoor deployment